- Some familiarity with the Haas control and with G-code programming is assumed.

- Definition - A macro is a form of sub-program that includes non-G-code commands.  It is typically a common operation that will be called many times.

·	Macro statements - Any non-G-code command.  Includes statements such as:  IF, WHILE, GOTO, math functions, and variables.  Haas requires that the macro parameter bit be set in order to interpret macro statements.  If it is not set, attempts to enter a macro statement during editing will result in an alarm.  If it is not set, macro statements within programs that are being loaded will be changed to comments.

·	Why? - A G-code program is rigid in structure.  It cannot be altered in mid-operation.  Offsets are the only things that can be used to alter machine path from one run to the next.  Macro's allow additional 'macro statements' that make it more flexible.

·	Macro Enable parameter bit – Parameter 57 bit 22.  This parameter allows for the entry of macro statements.  This parameter is used at the time of program entry not program execution.  If this parameter is turned off and a macro statement is entered, the control will ignore it.  If the program is loaded from a file, these statement will be converted to comments.  If the statement is loaded manually in MDI or edit modes, the control will give an alarm.

Problem #1:

- Getting started- Start-up and zero control.  Select program 9000 and switch to edit mode.  Press reset before running any programs.  Edit programs in edit mode.  Run programs from mem mode.  Each program will build on the previous ones.

- Go to the parameter page.  Find parameter 57 bit 22, “ENABLE MACRO”.  Set this parameter to ‘0’.

- Go to the program page.  Attempt to enter the following block to the end of the program: #1124=1.  What happens?  What messages/alarms show up?

- Go to the parameter page.  Find parameter 57 bit 22, “ENABLE MACRO”.  Set this parameter to ‘1’.

- Go to the program page.  Attempt to enter the same block: #1124=1.  What happens?  What messages/alarms show up?

- The program will look like this when finished:

O9000;

#1124=1;

G-codes:--

The following are G-codes that are of particular interest to the macro programmer.  They are available regardless of whether macro's are enabled.

M98 - Sub Program Call.  Pnnnn indicates the Onnnn of the program being called.  Lnn can be added to indicated the number of times the subprogram is to be called.  The subprogram must already be loaded into memory and must include an M99 to return to the main.  No arguments may be passed.

G65 - Macro subprogram call.  Allows arguments to be passed to subprogram.  Pnnnn indicates the Onnnn of the program being called.  Lnn can be added to indicated the number of times the subprogram is to be called.  Arguments are preceded with a command letter (A, B, etc.).  Arguments are copied to local variables #1-#26.

M97 - Local Sub Routine Call.  Pnnnn indicates beginning block number for subroutine.  Cannot branch to another program.

M96 - Conditional Local Branch when Discrete Input Signal is 0.  Qnn designates which input to test.  Pnnnn designates the block to branch to if condition is true.  Cannot branch to another program.  Not available in DNC.  Not available with cutter comp.  Stops Look ahead until after test is complete.

M00 - Stop Program.  Halts program.  Cycle start will continue at next block.  Shuts off TSC.

M01 - Optional Program Stop.  Same as M00 except that it won't stop if option stop is not turned on at front panel.

M30 - Program End and Rewind.  Stops program.  Returns cursor to beginning of program.  Stops spindle, shuts off all coolant, and cancels all tool length offsets.

M99 - Sub Program/subroutine Return or Loop.  Returns control to the main from a subroutine or subprogram.  If it is in the main, it will continually loop back to the beginning of the program.  If a Pnnnn is included, it performs an unconditional jump to the block indicated.  This last mode is similar to the GOTO statement.

G103 - Block Lookahead Limit.  Blocks are prepared well in advance in order to produce smooth motion.  The number of blocks of Look ahead can be limited.  G103 P0 or G103 disable limits.  G103 Pnnnn indicates the number of blocks allowed to be viewed in advance.  The actual number is Pnn + 2.  This means that the lower limit is 3 blocks. Not available with cutter comp.

G04 - Dwell.  Pnnn indicates the dwell time.  If there is a decimal point, the units are in seconds.  If no decimal, the units are in milliseconds.

Problem #2:

- Add 3 end-of-blocks (;) to the end of program O9000.  Run the program.  What happens to the cursor (highlighted block)?

- Add M00 immediately after the ‘#1124=1;’.  Run the program.  What happens to the cursor (highlighted block)? Switch to the diagnostics page and locate the M21 output.  Rerun the program.  What happens to the output?

- Replace M00 with M99.  Run the program.  What happens to the cursor (highlighted block)?  Switch to the diagnostics page and locate the M21 output.  Rerun the program.  What happens to the output?

- Replace M99 with M30.  Run the program.  What happens to the cursor (highlighted block)?  Switch to the diagnostics page and locate the M21 output.  Rerun the program.  What happens to the output?

- Delete the end-of-blocks at the end of the program. The program will look like this when finished:

O9000;

#1124=1;

M30;

Problem #3:

- Add ‘G04 P2.; #1124=0;’ immediately before the M30.  Switch to the diagnostics page and locate the M21 output.  Run the program and observe the status of this output.  What happened?  When did the output turn on and off?

- Add ‘G103 P1;’ immediately after the O9000.  Switch to the diagnostics page and locate the M21 output.  Run the program and observe the status of this output.  What happened?  When did the output turn on and off?

- Add 3 end-of-blocks (;) between the G103 P1 and the #1124=1.  Add another 3 end-of-blocks (;) between the G04 P2. and the #1124=0.  Switch to the diagnostics page and locate the M21 output.  Run the program and observe the status of this output.  What happened?  When did the output turn on and off?

- The program will look like this when finished:

O9000;

G103 P1;

;

;

;

#1124=1;

G04 P2.;

;

;

;

#1124=0;

M30;

·	Parentheses – Parentheses are used to enclose comments.  They indicate text that will not be executed as a part of the g-code program.  They are also used in conjunction with certain macro variables to provide the text for a programmable stop or alarm. Multiple levels of parentheses are not allowed (i.e. ((text)text) is not allowed ).   An opening parenthesis, (, must always have a close, ).

·	Comments are used to add notes to the program code.  They can describe revisions, document program flow in English, and even give a program a name.

·	With macro’s disabled, any block that contains a macro statement will be converted to comments.  The comment will begin with a question mark.  This will occur at the time the file is loaded into the control.  (i.e. (? #1=#1+#3)  )

·	Brackets – Brackets are used to control the order of execution of expressions within a g-code program.  Expressions within the innermost set of brackets are executed first.  Multiple levels of brackets are allowed (i.e. ABS[[3+1]*2] is acceptable).  An opening bracket, [, must always have a close, ].

·	Line Numbers – Line numbers are a way of assigning a label to a block.  Nnn may be placed anywhere on the line however it is typically in the beginning.  They are optional.  Line numbers can be used with sub-routine calls.  Nnn indicates the target of a M99 Pnn, M97 Pnn, or GOTOnn statement.

·	Aliasing – Aliasing is the act of assigning a name (G-code) to a specific program.  Macro's are typically a subprogram, not a stand-alone program.  They are called via G65 or M98.  This subprogram call can be replaced with a single M- or G-code.   The assignment of this new code to a program takes place throught parameters 81-100.  Only programs O9000 to O9019 may be aliased.

·	When aliased to a G-code, variables may be passed.  With an M-code, variables may not be passed.

·	This is particularly useful when assigning a function to an M-code.  (For example, M50 performs a pallet change on a vertical.  M50 is aliased to program O9001.)

Problem #4:

- Modify program O9000. Change the M30 to an M99.

- Start a new program, O0010.  Using any sub-program call (aliasing,  M98, or G65), write a line that will call program O9000.  End this program with an M30.

- Enable single block mode. (press single block button).

- Step through program O0010 using cycle start.  Is program O9000 called properly?  What happens when the M99 in program O9000 is reached?

·	Variables - Variables provide great flexibility in macro programming.  They can be changed manually as well as by the program itself.  Variables always numbered and follow a # sign.  E.g. #1, #1100, #33.  Variables can be a maximum of four digits.  Both local and global variable can be viewed and edited in the macro variable page of the current commands display.  Although system variables can be written to and read from, they cannot be viewed.  Parameters and settings cannot be accessed using macro variables.  Variables can replace any constant in a G-code command (e.g. G00 X#100).  Variables may be nested (e.g. #[#100]) to provide indirect addressing

·	Local variables  - Local variables are in the range of #1 to #33.  These are cleared after each G65 call.  Local variables are used to transfer data from the main to the macro (on the G65 line).  They can also be used as disposable storage locations.  Note that when passing variables, either normal or alternate addressing will be selected, not both.

·	In a source file, data can be passed to a subprogram.  See Appendix C.  The data is assigned to a variable depending upon the alphabetic code used.  For example, a G65 P12 X1. T3. is a sub-progarm call to program O0012.  As program O0010 is run, the X and T data will be available in variables #24 and #20 respectively.  If multiple IJK’s are present, then the alternate addressing is assumed.

·	Global variables - Global variables are in the range of #100-199 and #500-699.  These are not cleared with each G65 call.

·	System variables - System variables are always four digits.  They provide a way to interact with the machine operation.  They can be used to read and write to the discrete I/O.  They provide access to all offsets, positions, timers, programmable alarms,  and modal group codes.  See pages 163-167 for a more detailed description.

·	Macro variables can be viewed on the Current commands page.

·	Global macro variables are typically rounded in the last digit.  For example, a 5 may be stored as 4.999999.  This is particularly true of math operations.  The statement  6/2 may be evaluated as 3.000001.  6/2+6/2 could have a answer anywhere in the range 5.999998 to 6.000002.  If this value were then compared with 6, the answer may be true or false depending on the the rounding.  This must be accounted for when using these variables.  If a program is expecting a whole number, it is frequently useful to use the ROUND function described later.

·	Operators - Operators are symbols and commands that modify data.  They are classified as Arithmetic, Logical, and Boolean.  Results from an operator can be integers (or binary), floating point, or true(1)/false(0).

·	Arithmetic operators - Arithmetic operators perform simple math functions.  They are +, -, *, /, and MOD.  Plus and minus may also be used to indicate the sign of the data.  Arithmetic operators may not be used in conditional statements.  MOD performs a division and outputs the remainder.  */ are executed before +-.  x MOD y is the same as x/y® remainder.  (1+2, #1022-1)

·	Logical operators - Logical operators work on binary bit numbers.  When performed on a floating point number, only the integer part will be used.  These operators are OR, AND, and XOR.  (#100 AND #22, #500 XOR 4)

·	Boolean operators - Boolean operators are always evaluated as true(1) or false(0).  They are EQ, NE, GT, LT, GE, and LE.  (#100 EQ 0, #33 LE 4.5)

Problem #5:

- Turn off single block mode.

- Add a counter to program O9000.  This counter should increment by one, every time the program is run.  Use any variable in the range 100 to 199.

- A counter is a variable that increments by 1 every time the program is run.  They typically look like:  #100=#100+1.

- Run program O0010.  Observe the value of the counter variable.  Does it increment properly?

·	Functions - Functions are mathematical operations.  They are complex routines that simplify programming.

·	The functions are SIN, ASIN, COS, ACOS, TAN, ATAN, SQRT, ABS, ROUND, and FIX.

·	ABS returns the absolute value of the given decimal.

·	ROUND rounds off a decimal.

·	FIX returns the whole number portion of a fraction with no rounding.  If FIX is used in an arithmetic expression, it will round to the nearest whole number.  If it used as an address, the fraction is rounded to the addresses significant precision.  (i.e. X[ROUND[#1]] will round to 0.xxxx decimal places).  (SQRT[4.], COS[#100]).

·	DPRNT is a special function that allows data or text to be sent to the serial port.

·	Expressions - Expressions are defined as a sequence of variables and values surrounded by brackets [ ].

·	An entire expression can be assigned to a variable.  For example, suppose #100=10 and #510=3.  The expression #101= #[#100 +500] would load variable #101 with the value 3.  This method is very useful when working with look-up tables.

·	Arithmetic Expressions - Arithmetic Expressions produce a floating point number or integer.  They can be used as stand alone in the form of assignments or in conjunction with other G-codes.  (#3=[#3+1]*2, X[#501-#22], #[#2+2]=0)

·	Conditional Expressions - Conditional Expressions produce a value that is either true(1) or false(0).  They are used in conjunction with IF and WHILE.  (IF [#100 EQ 4.5] THEN GOTO20, #2=[#4 GT 9.2])

Problem #6:

- Modify program O0010 so that the sub-program call includes the ability to pass a variable.  Duplicate this block twice.  Use the values -2., -3., -4.5 for the passing variable in each block.

- Modify the O9000 program to include some X-axis moves.  Use ‘G00 G53 G90 X_’ for the move.  Fill in the blank for the location value/variable.

- Run program O0010.  Does the x-axis move to the correct spots?  Is the counter incrementing properly?

- Modify the O9000 program to include some Y-axis moves.  After the mill reaches the correct X location, copy this current location (#5021) to a macro variable in the range 100-199.  Use the value in this variable to move the Y-axis by the same amount.

- Run program O0010.  Do the X and Y-axes move to the correct spots?  Is the counter incrementing properly?

- If done properly, this exercise should produce a stair-stepped motion.  Did this occur?

·	Statements - Statements are commands that can utilize variables, operators, and/or expressions.

·	Assignment Statements - Assignment statements allow the user to change variables.  They always include an equals (=) sign.  When including arithmetic expressions, they must always be placed after the = sign.  The syntax is as follows:  #<variable>= <expression or variable>.  (#100=#100+1, #150=#5021)

·	Control Statements - Control statements allow the programmer to branch.  They control program flow both conditionally and unconditionally.

·	Unconditional Branch - An unconditional branch will always jump to a specified block in the current program.  It is illegal to branch from one program to another.  There are two methods of unconditional branch.  M99 Pnnnn is the most common way to unconditional branch.  GOTOnnnn is the same as M99 except that it may be placed on the same line as other G-codes.  The GOTO statement can also include arithmetic expressions and variables.  If the computed expression or variable includes a fraction, the nearest whole number will be used by GOTO.  (GOTO3, GOTO#100, GOTO[#511+#22])

·	Conditional Branch/Execution - IF [<conditional statement>] THEN <statement> is the command used for conditional branches and execution.  The <statement>  is executed if the <conditional statement> is evaluated as true(1).  The <statement> can be an M99, GOTO, assignment statement, or G-code.  THEN is optional.  If the <statement> is an M99 or GOTO, then the IF is also optional.  (IF [#1022 EQ 0] THEN GOTO3, IF [[#1024] OR [#1022 EQ 0]] #115=#115+1, [#1000] GOTO10)

·	Looping - Traditional G-code allows looping using the L address.  This will only allow a fixed number of loops.  The WHILE statement allows for looping based on conditions.  The format is:

WHILE [<conditional expression>] DOn

<statements>

ENDn

WHILE can be abbreviated WH.  The DOn-ENDn are a matched pair.  The value of n is limited to 1..3 due to a maximum of three nested loops per subroutine.  The statements will be executed until the expression if evaluated as false.  Eliminating the WHILE  will result in an infinite loop.  (WHILE [#1022 EQ 0] DO1; #100=#100+1; END1,  WH [#[#33+1000] EQ 1] DO1; WH [#2 GT 4.] DO2; END2; END1)

Problem #7:

- Reset the counter variable to zero (from problem #5).

- Delete 2 of the subprogram calls from program O0010.  Replace them with a G28

- Modify the O0010 program so that it will run for a specific number of times. Use IF or WHILE to determine if this number of times has been exceeded.  The program should stop after the program has run for the specified number of times.  Use the value 5 for the number of times to be run.  It is ok to use the counter in program O9000.

- Run program O0010.  Was program O9000 performed the correct number of times?

·	System Variables – There are certain macro variables that are not available for general pupose use.  These variabls have been assigned a specific function within the Haas contol.  Some of these variables are read only.  These variables allow program access to certain pieces of data within the control.  These include such things as current machine position and parameters.

·	System variables cannot be viewed directly.

Partial Variable Listing:

| Variable # | Function |
| --- | --- |
| #0 | Not a number (read only) |
| #1-#33 | Macro call arguments (local variables) |
| #100-#199 | General purpose variables saved on power off (global variables) |
| #1000-#1063 | 64 discrete inputs (read only) |
| #1100-#1155 | 56 discrete outputs |
| #3000 | Programmable alarm |
| #3001 | Millisecond timer |
| #3002 | Hour timer |
| #3004 | Override control |
| #3006 | Programmable stop with message, 15 characters max |
| #4101-#4126 | Previous block address codes |
| NOTE: Mapping of 4101 to 4126 is the same as the alphabetic addressing of “Macro Arguments section”; e.g. the statement X1.3 sets variable #4124 to 1.3. |  |
| #5021-#5025 | Present machine coordinate position |
| NOTE: Variables are listed in XYZABCUVW order. |  |
| #6501-#6999 | Parameters (read only) |

Problem #8:

- Reset the counter variable to zero (from problem #4).

- Modify the O0010 program so that the entire cycle (all 5 times) is timed.  Store the value of the timer in a variable in the range 100-199.  At the end of the cycle, display the message “cycle complete”.

- Run program O0010.  How long did it take for the cycle?  Was the message displayed?

·	Settings – Certain settings can effect how programs are run.  Programs O9xxxx are typically used for macro’s although any program can reside here.  These settings effect all O9xxxx programs as a group.

·	Edit lock prohibits the editing of any O9xxxx program.  It also hides these programs in memory.  If all programs are saved, the hidden programs will not be included.

·	Trace allows for the program to be viewed during execution.  This setting is useful for troubleshooting.  While disabled, the control will wait with the message “running” while the macro is executed.

·	Allows for the use of single block.  If this setting is turned off, the entire macro will be treated as a single block.  Single block mode will have no effect on the macro execution.

Problem #9:

- Delete programs O0010 and O9000 as they will no longer be needed.  You may pick either Problem #9 or Problem #10.

- You are the maker of custom dog dishes.  All dog dishes are rectangular.  They can come in any size.  They can be made from any one of several materials.  Your machinist does not have a print and does not know how to program in G-code.  He can only operate the mill.  Provide a safety factor that will not allow the tool to be run into the table.  The operator must be prompted to change parts after each part is complete.  The operator must be notified when the run is complete.  Air blast (#1124) is used to blast chips away.  It should only be used during cuts, not rapids.  Create a program that will make these parts.  The operator is only given the dimensions (XxYxZ), material type, and quantity of parts.  The operator is not allowed to change the g-code program when switching jobs.

- For this exercise, ignore cutter diameter/comp.  You do not have to mill out the middle of the dish, only the perimeter.  An empty tool holder will crash into the table at a distance of  4.5 inches from machine zero.

- Make two runs of parts.  Run #1- 2x2x3, plastic, 3 parts.  Run #2- 0.5x0.5x0.3, steel, 2 parts.

- Material specs:		Plastic			Aluminum		Steel

Spindle speed:		3000 rpm		2000			100

Z increment:		0.550”			0.245			0.09

feedrate:			300 in/min		160			25

tool #:			1			2			3

tool length:		1.”			1.375			1.75

- Useful G-codes:

G00-rapid

G01-feed

G53-offset for zero

G90-absolute

G91-incremental

M03 Snn-spindle forward

M05-spindle stop

M06 Tnn- tool change

Problem #10:

- Delete programs O0010 and O9000 as they will no longer be needed.  You may pick either Problem #9 or Problem #10.

- You are an engineer developing a new tool changer for the mill.  The new tool changer has a unique carousel.  The carousel has been added to the software using M39 Tnn.  You need to run a complete tool change in order to test out the shuttle mechanism.  The shuttle has not been integrated with the new M39 yet, so you must create a macro to integrate it.  The shuttle works in the same fashion as a standard VF tool changer.

- Be sure to include safety factors such as protecting the motor from an indefinite stall, and alarming in an invalid state.  If possible, check for broken/stuck switches.  Use M06 Tnn to call the tool change.  Be sure to include all of the spindle movements.

- Write a macro to run the tool changer of a vertical to simulate the M06.  Use M39 Tnn for all carousel moves.

- Useful G-codes and variables:

M19-spindle orient

M82-tool unclamp

M86-tool clamp

#1000-shuttle in switch

#1001-shuttle out switch

#1108-shuttle in motor

#1109-shuttle out motor

# Appendix A - G-codes

The following is a summary of the G codes.

| Code: | Function: |
| --- | --- |
| G00 | Rapid Motion |
| G01 | Linear Interpolation Motion |
| G02 | CW Interpolation Motion |
| G03 | CCW Interpolation Motion |
| G04 | Dwell |
| G09 | Exact Stop |
| G10 | Set Offsets |
| G12 | CW Circular Pocket Milling (Yasnac) |
| G13 | CCW Circular Pocket Milling (Yasnac) |
| G17 | XY Plane Selection |
| G18 | ZX Plane Selection |
| G19 | YZ Plane Selection |
| G20 | Select Inches |
| G21 | Select Metric |
| G28 | Return To Reference Point |
| G29 | Return From Reference Point |
| G31 | Feed Until Skip (optional) |
| G35 | Automatic Tool Diameter Measurement (optional) |
| G36 | Automatic Work Offset Measurement (optional) |
| G37 | Automatic Tool Offset Measurement (optional) |
| G40 | Cutter Comp Cancel |
| G41 | 2D Cutter Compensation Left |
| G42 | 2D Cutter Compensation Right |
| G43 | Tool Length Compensation + |
| G44 | Tool Length Compensation - |
| G47 | Text Engraving |
| G49 | G43/G44/G143 Cancel |
| G50 | G51 Cancel |
| G51 | Scaling (optional) |
| G52 | Set Work Coordinate System G52 (Yasnac) |
| G52 | Set Local Coordinate System (Fanuc) |
| G52 | Set Local Coordinate System (HAAS) |
| G53 | Non-Modal Machine Coordinate Selection |
| G54 | Select Work Coordinate System 1 |
| G55 | Select Work Coordinate System 2 |
| G56 | Select Work Coordinate System 3 |
| G57 | Select Work Coordinate System 4 |
| G58 | Select Work Coordinate System 5 |
| G59 | Select Work Coordinate System 6 |
| G60 | Unidirectional Positioning |
| G61 | Exact Stop Modal |
| G64 | G61 Cancel |
| G65 | Macro Subroutine Call (optional) |
| G68 | Rotation (optional) |
| G69 | G68 Cancel (optional) |
| G70 | Bolt Hole Circle (Yasnac) |
| G71 | Bolt Hole Arc (Yasnac) |
| G72 | Bolt Holes Along an Angle (Yasnac) |
| G73 | High Speed Peck Drill Canned Cycle |
| G74 | Reverse Tap Canned Cycle |
| G76 | Fine Boring Canned Cycle |
| G77 | Back Bore Canned Cycle |
| G80 | Canned Cycle Cancel |
| G81 | Drill Canned Cycle |
| G82 | Spot Drill Canned Cycle |
| G83 | Normal Peck Drill Canned Cycle |
| G84 | Tapping Canned Cycle |
| G85 | Boring Canned Cycle |
| G86 | Bore/Stop Canned Cycle |
| G87 | Bore/Stop/Manual Retract Canned Cycle |
| G88 | Bore/Dwell/Manual Retract Canned Cycle |
| G89 | Bore/ Dwell Canned Cycle |
| G90 | Absolute |
| G91 | Incremental |
| G92 | Set Work Coordinates - FANUC or HAAS |
| G92 | Set Work Coordinates - YASNAC |
| G93 | Inverse Time Feed Mode |
| G94 | Feed Per Minute Mode |
| G98 | Initial Point Return |
| G99 | R Plane Return |
| G100 | Cancel Mirror Image |
| G101 | Enable Mirror Image |
| G102 | Programmable Output To RS-232 |
| G103 | Limit Block Buffering |
| G107 | Cylindrical Mapping |
| G110 | Select Work Coordinate System 7 |
| G111 | Select Work Coordinate System 8 |
| G112 | Select Work Coordinate System 9 |
| G113 | Select Work Coordinate System 10 |
| G114 | Select Work Coordinate System 11 |
| G115 | Select Work Coordinate System 12 |
| G116 | Select Work Coordinate System 13 |
| G117 | Select Work Coordinate System 14 |
| G118 | Select Work Coordinate System 15 |
| G119 | Select Work Coordinate System 16 |
| G120 | Select Work Coordinate System 17 |
| G121 | Select Work Coordinate System 18 |
| G122 | Select Work Coordinate System 19 |
| G123 | Select Work Coordinate System 20 |
| G124 | Select Work Coordinate System 21 |
| G125 | Select Work Coordinate System 22 |
| G126 | Select Work Coordinate System 23 |
| G127 | Select Work Coordinate System 24 |
| G128 | Select Work Coordinate System 25 |
| G129 | Select Work Coordinate System 26 |
| G136 | Automatic Work Offset Center Measurement |
| G141 | 3D+ Cutter Compensation |
| G143 | 5 AX Tool Length Compensation (optional) |
| G150 | General Purpose Pocket Milling |
| G174/184 | General-purpose Rigid Tapping |
| G187 | Accuracy Control for High Speed Machining |

# Appendix B - M-codes

Only one M code may be programmed per block of a program. All M codes are effective or cause an action to occur at the end of the block. However, when Parameter 278 bit "CNCR SPINDLE" is set to 1, an M03 (spindle start) will occur at the beginning of a block.

| Code: | Function: |
| --- | --- |
| M00 | Stop Program |
| M01 | Optional Program Stop |
| M02 | Program End |
| M03 | Spindle Forward |
| M04 | Spindle Reverse |
| M05 | Spindle Stop |
| M06 | Tool Change |
| M08 | Coolant On |
| M09 | Coolant Off |
| M10 | Engage 4th Axis Brake |
| M11 | Release 4th Axis Brake |
| M12 | Engage 5th Axis Brake |
| M13 | Release 5th Axis Brake |
| M16 | Tool Change (same as M06) |
| M19 | Orient Spindle |
| M21 | M28 Optional Pulsed User M Function with Fin |
| M30 | Prog End and Rewind |
| M31 | Chip Conveyor Forward |
| M32 | Chip Conveyor Reverse |
| M33 | Chip Conveyor Stop |
| M34 | Increment Coolant Spigot Position |
| M35 | Decrement Coolant Spigot Position |
| M36 | Pallet Rotate |
| M39 | Rotate Tool Turret |
| M41 | Low Gear Override |
| M42 | High Gear Override |
| M50 | Execute Pallet Change |
| M51-M58 | Set Optional User M |
| M61-M68 | Clear Optional User M |
| M75 | Set G35 or G136 Reference Point |
| M76 | Disable Displays |
| M77 | Enable Displays |
| M78 | Alarm if skip signal found |
| M79 | Alarm if skip signal not found |
| M82 | Tool Unclamp |
| M86 | Tool Clamp |
| M88 | Through the Spindle Coolant ON |
| M89 | Through the Spindle Coolant OFF |
| M95 | Sleep Mode |
| M96 | Jump if no Input |
| M97 | Local Sub-Program Call |
| M98 | Sub Program Call |
| M99 | Sub Program Return Or Loop |

# Appendix C – Alphabetic Addressing

Alphabetic Addressing

| Address: | A | B | C | D | E | F | G | H | I | J | K | L | M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Variable: | 1 | 2 | 3 | 7 | 8 | 9 | - | 11 | 4 | 5 | 6 | - | 13 |
| Address: | N | O | P | Q | R | S | T | U | V | W | X | Y | Z |
| Variable: | - | - | - | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 |

Alternate Alphabetic Addressing

| Address: | A | B | C | I | J | K | I | J | K | I | J | K |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Variable: | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
| Address: | I | J | K | I | J | K | I | J | K | I | J | K |
| Variable: | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| Address: | I | J | K | I | J | K | I | J | K |  |  |  |
| Variable: | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |  |  |  |

# Appendix D – Macro Variables

| Variable # | Function |
| --- | --- |
| #0 | Not a number (read only) |
| #1-#33 | Macro call arguments (local variables) |
| #100-#199 | General purpose variables saved on power off |
| #500-#699 | General purpose variables saved on power off |
| #700-#749 | Hidden variables for internal use only. |
| #800-#999 | General purpose variables saved on power off |
| #1000-#1063 | 64 discrete inputs (read only) |
| #1080-#1087 | Raw analog to digital inputs (read only) |
| #1090-#1098 | Filtered analog to digital inputs (read only) |
| #1094 | Spindle load with OEM spindle drive (read only) |
| #1098 | Spindle load with Haas vector drive (read only) |
| #1100-#1155 | 56 discrete outputs |
| #2000-#2199 | Tool length offsets |
| #2201-#2399 | Tool length wear |
| #2401-#2599 | Tool diameter/radius offsets |
| #2601-#2799 | Tool diameter/radius wear |
| #3000 | Programmable alarm |
| #3001 | Millisecond timer |
| #3002 | Hour timer |
| #3003 | Single block suppression |
| #3004 | Override control |
| #3006 | Programmable stop with message |
| #3011 | Year, month, day |
| #3012 | Hour, minute, second |
| #3020 | Power on timer (read only) |
| #3021 | Cycle start timer (read only) |
| #3022 | Feed timer (read only) |
| #3023 | Present part timer (read only) |
| #3024 | Last complete part timer (read only) |
| #3025 | Previous part timer (read only) |
| #3026 | Tool in spindle (read only) |
| #3027 | Spindle RPM (read only) |
| #3901 | M30 count 1 |
| #3902 | M30 count 2 |
| #4000-#4021 | Previous block group codes |
| #4101-#4126 | Previous block address codes |
| NOTE: Mapping of 4101 to 4126 is the same as the alphabetic addressing of “Macro Arguments section”; e.g. the statement X1.3 sets variable #4124 to 1.3. |  |
| #5001-#5005 | Previous block end position |
| #5021-#5025 | Present machine coordinate position |
| #5041-#5045 | Present work coordinate position |
| #5061-#5064 | Present skip signal position |
| #5081-#5085 | Present tool offset |
| #5201-#5205 | Common offset |
| #5221-#5225 | G54 work offsets |
| #5241-#5245 | G55 work offsets |
| #5261-#5265 | G56 work offsets |
| #5281-#5285 | G57 work offsets |
| #5301-#5305 | G58 work offsets |
| #5321-#5325 | G59 work offsets |
| #5401-#5500 | Tool feed timers (seconds) |
| #5501-#5600 | Total tool timers (seconds) |
| #5601-#5699 | Tool life monitor limit |
| #5701-#5800 | Tool life monitor counter |
| #5801-#5900 | Tool load monitor maximum load sensed so far |
| #5901-#6000 | Tool load monitor limit |
| #6001-#6277 | Settings (read only) |
| #6501-#6999 | Parameters (read only) |
| NOTE: The low order bits of large values will not appear in the macro variables for settings and parameters |  |
| #7001-#7005 | G110 additional work offsets |
| #7021-#7025 | G111 additional work offsets |
| #7041-#7045 | G112 additional work offsets |
| #7061-#7065 | G113 additional work offsets |
| #7081-#7085 | G114 additional work offsets |
| #7101-#7105 | G115 additional work offsets |
| #7121-#7125 | G116 additional work offsets |
| #7141-#7145 | G117 additional work offsets |
| #7161-#7165 | G118 additional work offsets |
| #7181-#7185 | G119 additional work offsets |
| #7201-#7205 | G120 additional work offsets |
| #7221-#7225 | G121 additional work offsets |
| #7241-#7245 | G122 additional work offsets |
| #7261-#7265 | G123 additional work offsets |
| #7281-#7285 | G124 additional work offsets |
| #7301-#7305 | G125 additional work offsets |
| #7321-#7325 | G126 additional work offsets |
| #7341-#7345 | G127 additional work offsets |
| #7361-#7365 | G128 additional work offsets |
| #7381-#7385 | G129 additional work offsets |