Stand-Alone BIOS Flash4 Program  Command line Interface

Sep 21, 1999

The DOS-based flash4 engine is a command line utility. The engine runs in a "silent" mode and requires no user interaction. All parameters are passed in from command line. The name of the executable is "flash4.exe".

The usage of the utility is as follows.

Flash4.exe [-version] [-scan] [-bus #] [-dev #] [-1mb] [-r] [-s] [-w] [-hv] [–bv] [-checksum] <romimage>

Flash4.exe	Report to console the usage of the utility. All other options are ignored.

Flash4.exe -?	Report to console the usage of the utility. All other options are ignored.

Flash4.exe /?	Report to console the usage of the utility. All other options are ignored.

Flash4.exe –version		Report to console, the version number of the program, and exit immediately.  All other command line options are ignored.

Flash4.exe -scan		Scans all the Adaptec SCSI devices and displays the information about them.

Flash4.exe -r [-bus #] [-dev #] 	Read the current BIOS. Calculate & display the checksum.

The bus # and device # can be specified in hex also. To specify them in hex append 0x to the hex number.

Example:

To specify bus no. 0 and device no. 0f (in hex),

Flash4.exe -bus 0 –dev  0x0f ...

Flash4.exe -s [-bus #] [-dev #] <romimage>

Read the current BIOS and save it in the file and calculates the checksum.  All other options are ignored.

If a filename is specified, save the contents to the file. Otherwise, use the default filename "8bios.com".

The bus # and dev # can be specified in hex also. To specify them in hex append 0x to the hex number.

Example :

To specify bus num 0 and dev num 0f ( in hex ),

Flash4.exe -bus 0 -dev 0x0f ...

Flash4.exe [-bus #] [-dev #]  -w <romimage>

Update the BIOS with the image specified in the file.  Always read the BIOS back after the flash and report error if the read-back image does not match the file.

If "-bus" or "-dev" are specified, update only the matched adapter.

By default, the program updates all the adapters that matches the Vendor ID and Device ID in the new BIOS image.

Flash4.exe [-checksum] <romimage>

Calculate the checksum of the file specified in the argument and display the same.

Flash4.exe [-bus #] [-dev #] –hv

Displays hardware version information about the host adapter.

Flash4.exe [-bus #] [-dev #] –bv

Displays BIOS version information about the BIOS on the host adapter.

Flash4.exe –hv <romimage>

Displays hardware version information embedded in the supplied romimage file.

Flash4.exe –bv <romimage>

Displays BIOS version information embedded in the supplied romimage file.

Flash4.exe –1mb < -r | -s >  <romimage>

When used in conjunction with –r or –s, it will allow reading and saving of 1Mbit ROM Image. This switch is not necessary when performing a checksum on a 1Mbit BIOS image file.

Note: -1mb is supported inherently for –w. In other words, it will write 1Mbit if the romimage file is 128KB.

When no arguments are specified to the flash4 utility. It just displays the usage of the utility.

The arguments can appear in any order.  If an argument requires a parameter, it must appear immediately after the argument.  For example, a valid number must follows, immediately either the "-dev" or "-bus" option.

Unless the program receives the "-verbose" option, it prints.  Otherwise, it reports the status of the execution.

The "-version" option has the highest precedence.  If this option is present, all others are ignored.  The -scan option has the next precedence. The "-r" option has the precedence after that.  The program will read the current BIOS when this option is present.

The "-verbose" option does not apply if the "-version" option is present.

Upon exit, the program returns the status of the execution.  The possible return values are listed in the table below.

Status Code	Meaning

0		Program ran successfully.

1		Command line syntax error.

Possible Cause is Invalid option at the command line

2		Invalid parameters following the argument.

- File System error.

Reasons:

- Can not open BIOS image file.

- Can not read from or write to the specified file.

4		Invalid bios image file

Reasons :

- Bios image size may be incorrect. We assume 64k or 128k bios image size.

- Bios signature (55AA as first two bytes) may be incorrect.

- Bios image size may be greater than installed expansion ROM.

5		Unable to determine PCI Bios support. The underlying hardware may not be 			supporting PCI interface.

6		Memory allocation error. Unable to allocate sufficient memory.

- Expansion ROM is not present.

The adapter card doesn't have an expansion ROM.

8		Verification failed. Unable to verify the updated Expansion ROM image.

9		Unable to read or write the configuration registers. Probable cause may be 			problem with a 	PCI Bios interrupt.

10		Device not existent.  Cannot locate any Adaptec device. No Adaptec SCSI 			devices are present on the PCI bus.
11		File System write error. Insufficient disk space.

- Unable to flash any of the host adapter

- Unable to flash some of the host adapters

- Unable to retrieve information for BIOS version less than 2.20.

Additional Notes:

1)  Flash4 utility can not run when “EMM386.EXE” is active. Please disable “EMM386.EXE” by running “EMM386.EXE OFF” at the DOS prompt before running flash4.