CamView 3D

Interactive Demonstration Version

Release 1.10

THIS COMPUTER SOFTWARE PROGRAM, DOCUMENTATION AND SAMPLE DATA FILES

ARE PROTECTED UNDER THE COPYRIGHT LAWS OF THE UNITED STATES OF AMERICA.

COPYRIGHT 1993-1994  --  ALL RIGHTS RESERVED.

Robert A. Pemberton  provides no warranties, either  expressed or

implied,  of merchant ability,  or fitness, for a particular  use

or purpose of this SOFTWARE and documentation.  In no event shall

Robert A. Pemberton be held liable for any damages resulting from

the use or misuse of the SOFTWARE and documentation.

_______________________________________________________________________________

Welcome to the Interactive Shareware Demo of CamView 3D

The purpose of  this demonstration is  to  show  you the  features  and

capability of the  CamView 3D system  while offering the opportunity to

try  the program with your own  CNC programs.  This  demo is  identical

to the registered version of the program except for the following items.

1) The maximum file length which can be processed is 100k.

2) Printing and command line file specification has been disabled.

3) The program will only run 1 file at a time. User must exit to DOS between

files.

4) This message is displayed on startup.

We hope you will find this program useful.  Please see the included file

REGISTER.DOC for registration and price information.

________________________________________________________________________________

Whats New?

Configuration File:

This release of CamView 3D provides a configuration utility which allows

the user to customize several initial aspects of the program.

The configuration screen is accessed by pressing <F10> from the file

selection menu.  The default settings are stored in a file named

CAMVIEW.CFG which is located in the current CamView directory.

Default Suffix:

The 3 character suffix may be changed to suit users needs.

Axis Display:

The axis display may be toggled on and off.

Arc Resolution:

The arc resolution may be changed.  Entering smaller values will result

in all lines and arcs being drawn with greater resolution but reduced

processing speed.  Larger values will decrease drawing resolution and

provide greater proceessing speed.

Default Delay:

The line to line delay value may be adjusted to better simulate the

machining process.

Relaxed Demo Version Restrictions:

This Demo version of CamView 3D will now read any valid DOS filename.

Bug Fix:

Several minor bugs have been eliminated and error checking and recovery

has been enhanced.

________________________________________________________________________________

What is CamView?

CamView is a CNC (Computer Numeric Control)  control file viewing system which

allows the user to view control files for many types  of CNC equipment in full

3D space  on your  desktop PC.  This allows  you  to quickly  troubleshoot and

verify your control  programs without holding up your expensive machine  tools

for "dry runs".  CamView 3D is also a great teaching tool, enabling the novice

programmer  to visualize and  verify their programs  easily.  CamView  makes a

great companion to any existing CAD/CAM programming system and is very helpful

in maintaining a CNC program library.

CamView was written for programs following the Bridgeport BOSS 10(tm)

programming language.  The following functions are supported.

G0 Rapid Traverse

G1 Linear Interpolation (feed)

G2 Circular Interpolation Clockwise

G3 Circular Interpolation Counterclockwise

All circular movement must be specified using the XYIJ format with I & J being

the X and Y axis center points.

_______________________________________________________________________________

SYSTEM REQUIREMENTS:

80386, 80486 or Pentium(tm) based system.

Hard drive.

VGA card and monitor with 640 x 480 x 16 resolution capability.

Math Coprocessor in 386 systems is recommended.

A read/write disk cache, such as Microsofts SMARTDRV(tm) will significantly

increase performance.  The cache should be 512k or greater.

_______________________________________________________________________________

INSTALLATION:

Copy the distribution files to any directory on your hard drive which is

included in the DOS PATH= statement.

_______________________________________________________________________________

RUNNING CamView 3D:

Upon running the CVIEW-3D program, the user is presented with a listing of

all .NCC files in the current directory.  The cursor movement keys are used

to highlight the desired file.  Pressing ENTER will load the file into the

viewing area.

The desired input file may also be specified at the command line as follows:

CAMVIEW filename.ncc   (Disabled in demonstration version)

This allows CamView 3D to be called from a batch file or file utility such

the File Manager in Microsoft Windows(tm).

_______________________________________________________________________________

RUNNING UNDER MICROSOFT WINDOWS (tm):

CamView 3D will run as a DOS based application in full screen mode under

Microsoft Windows 3.1x

______________________________________________________________________________

|       |                  |         |

|       |                  |         |

COMMANDS:  ROTATION                      |       |  NUMERIC KEYPAD  |         |

|       |__________________|         |

There are nine preset views included    |                                    |

for easy navigation.                    |                 R                  |

|                 e                  |

<1> Displays Left Front 3D View.        |                 a                  |

|                 r                  |

<2> Displays Front 2D View.             |                                    |

|   Left Rear     |     Right Rear   |

<3> Displays Right Front 3D View.       |             \       /              |

|                                    |

<4> Displays Left Side 2D view.         |              7  8  9               |

|                                    |

<5> Displays Top 2D View.               |  Left Side - 4  5  6 - Right Side  |

|                                    |

<6> Displays Right Side 2D View.        |              1  2  3               |

|                                    |

<7> Displays Left Rear 3D View.         |             /       \              |

|   Left Front    |     Right Front  |

<8> Displays Rear 2D View.              |                                    |

|                 F                  |

<9> DIsplays Right Rear 3D View.        |                 r                  |

|                 o                  |

|                 n                  |

|                 t                  |

|                                    |

|____________________________________|

In addition to the preset views, the toolpath may be rotated using the cursor

movement keys as follows.

<CTRL>+<LEFT ARROW>  Rotates toolpath around X axis in positive direction.

<CTRL>+<DOWN ARROW>  Rotates toolpath around Y axis in positive direction.

<CTRL>+<RIGHT ARROW> Rotates toolpath around Z axis in positive direction.

<ALT>+<LEFT ARROW>   Rotates toolpath around X axis in negative direction.

<ALT>+<DOWN ARROW>   Rotates toolpath around Y axis in negative direction.

<ALT>+<RIGHT ARROW>  Rotates toolpath around Z axis in negative direction.

User may also enter the X, Y, and Z rotation values from the keyboard by

pressing the appropriate axis key.  You will then be prompted to enter

the desired value (0 thru 360) for each axis.

_______________________________________________________________________________

COMMANDS: PAN

The toolpath may be shifted (PANNED) in four directions relative to the

the currently displayed view as follows.

<LEFT ARROW>  Shifts toolpath to the LEFT.

<DOWN ARROW>  Shifts toolpath DOWN.

<UP ARROW>    Shifts toolpath UP.

<RIGHT ARROW> Shifts toolpath to the RIGHT.

_______________________________________________________________________________

COMMANDS: ZOOM

The toolpath may be enlarged and reduced (ZOOMED) relative to the currently

displayed view as follows.

<+> Zooms toolpath IN.

<-> Zooms toolpath OUT.

_______________________________________________________________________________

DELAY:

You may specify a line to line delay factor for viewing files.  This allows

the user to visualize the exact order in which the cutting and rapid moves

will occur.

Pressing <D> while in the viewing screen will prompt user for a delay value.

This value may range from 0 (default) to 5 in increments of .1.  If you find

the delay value you have entered is too large, pressing <D> again will allow

you to enter the value again.

_______________________________________________________________________________

COMMANDS: MISC.

<F1>    Displays navigation help screen.

<F10>   Prints current display to printer. (Disabled in demonstration version)

<SPACE> Returns to file selection menu.    (Disabled in demonstration version)

<ESC>   Exits program.

_______________________________________________________________________________

AXIS DISPLAY:

The X, Y, and Z axis is displayed on screen as 3 blue lines running from the

maximum to minimum values for each axis and intersecting at the program

origin (X=0, Y=0, Z=0).  These are provided for reference only and help the

user identify where the toolpath is located in 3D space.

_______________________________________________________________________________

ABOUT THE INPUT FILES:

The default file extension for input files is .NCC and cannot be changed.

As CamView 3D begins reading the input file, it will skip over any lines

beginning with or containing the characters < ; > and < ' >.  These lines

are considered to be comments by the system and will be displayed on the

screen as the file is read.  The comments may contain any valid ASCII

character and used to identify such things as program origination, tooling

information, etc.  Any unsupported functions such as M codes are also

displayed in with the trigger character highlighted in yellow.

LINE NUMBERS:

Line numbers (Nxxxx) are not required and are ignored by CamView 3D.

_______________________________________________________________________________

TO SOFTWARE DISTRIBUTORS AND BBS SYSTEM OPERATORS:

The original CAMVIEW package is distributed in ZIP archive format. If you

wish to package it using a different archive routine (PAK,ARJ,ZOO,ARC,

self-extracting EXE, etc.) you may do so if you agree to follow these rules:

1. Under no circumstances change any of the CAMVIEW files in any way.

2. Do not remove any CAMVIEW files from the package.

3. Do not add other files to the package.

4. Keep the same name ( CV3D_101.ZIP )

5. CAMVIEW may be included as part of a Shareware CD-ROM package provided it

is made clear that the program is indeed Shareware and purchaser is still

required to register if they wish to continue using the program.

Your cooperation will be appreciated. You may also add archive file

"advertisements" and file comments as you wish as long as they do not

break any of the above rules.

_______________________________________________________________________________

REGISTRATION:

The CamView 3D package is priced as follows.

$80.00 - Single User Copy

A 10% discount may be applied on orders of 10 or more copies.

A 15% student discount may be applied if proof of current enrollment is

provided. (photocopy of student ID)

In return for your registration, you will receive the latest fully functional

copy of CamView 3D.  You will also be placed on our mailing list and notified

of all new releases and product developments.  You may also specify the

default file suffix for your copy of CamView 3D if .NCC is not acceptable.

_______________________________________________________________________________

BONUS: Included with all registrations is a free copy of the CNC2DXF.EXE

file conversion utility.  CNC2DXF will read any file compatible

with CamView 3D and produce a .DXF file which may be imported

into any software package supporting the DXF format.

The CNC2DXF utility may be purchased separately for $40.00

_______________________________________________________________________________

Please see the included file REGISTER.DOC for ordering information.

Quotes for custom versions available upon request.

Robert A. Pemberton

2524 Ashbrooke Drive

Lexington, KY  40513

E-Mail Sites:

America OnLine  -  RPEMBERTON

Internet        -  RPemberton@aol.com

_______________________________________________________________________________

CamView is in a constant state of development.

If you have any suggestions on how we can improve the software, please

drop me a note at any of the above addresses.

_______________________________________________________________________________